# HPE OneView PowerShell Module Version Compatibility

## Table of Contents

- Quick Selection Guide

  - For OneView 8.x+ appliances (recommended)

  - For OneView 7.x appliances (legacy)

- Installation Commands

- Connection Command

- How the Automation Selects Modules

- Module Validation

- Related Documentation

Project standard: This automation uses HPEOneView.1000 and requires PowerShell 7+ (it does not support Windows PowerShell 5.1). The pre-8.0 rows below are historical only and not used here.

| Module Name | PowerShell | .NET Standard | OneView Appliance Min | Notes |
| --- | --- | --- | --- | --- |
| HPEOneView.1000 | 7.0+ | 2.1 | 10.00 | Latest. Requires PowerShell 7+, .NET Standard 2.1 |
| HPRESTCmdlets | 5.1, 7.0+ | N/A | 1.0+ | HPE iLO REST/Sso. Windows-only. Used for virtual media and power operations. Vendored offline in scripts/modules/. |
| HPEOneView.910 | 7.0+ | 2.0 | 9.10 | PowerShell Core support |
| HPEOneView.900 | 7.0+ | 2.0 | 9.00 | PowerShell Core support |
| HPEOneView.860 | 7.0+ | 2.0 | 8.60 | PowerShell Core support |
| HPEOneView.840 | 7.0+ | 2.0 | 8.40 | PowerShell Core support |
| HPEOneView.830 | 7.0+ | 2.0 | 8.30 | PowerShell Core support |
| HPEOneView.800 | 7.0+ | 2.0 | 8.00 | PowerShell Core support |
| HPEOneView.720 | 5.1, 7.0+ | 2.0 | 7.20 | Last supporting PS 5.1 |
| HPEOneView.710 | 5.1, 7.0+ | 2.0 | 7.10 | Last supporting PS 5.1 |
| HPEOneView.700 | 5.1, 7.0+ | 2.0 | 7.00 | Last supporting PS 5.1 |

Important: HPOneView.Managed is NOT a standard HPE OneView module name. Use HPEOneView.1000, HPEOneView.900, etc.

## Quick Selection Guide

### For OneView 8.x+ appliances (recommended)

Install-Module HPEOneView.1000 -Scope AllUsers

### For OneView 7.x appliances (legacy)

Install-Module HPEOneView.720 -Scope AllUsers

## Installation Commands

Install-Module HPEOneView.1000 -Scope CurrentUser          # current user
Install-Module HPEOneView.1000 -Scope AllUsers             # all users (elevation)
Save-Module HPEOneView.1000 -Path C:\temp\oneview-modules  # offline
Import-Module HPEOneView.1000

Only ONE HPE OneView module version can be installed at a time. To switch:

Uninstall-Module HPEOneView.1000 -Force -ErrorAction SilentlyContinue
Uninstall-Module HPEOneView.900 -Force -ErrorAction SilentlyContinue
Install-Module HPEOneView.1000 -Scope CurrentUser -AllowClobber -Force
Save-Module HPEOneView.1000 -Path C:\temp\modules -Force

If multiple versions exist you may see Connect-OVMgmt: The term 'Connect-OVMgmt' is not recognized or cmdlet-name conflicts.

## Connection Command

All module versions use the same connection pattern:

Connect-OVMgmt -Hostname oneview.example.com -Credential $cred
# -Appliance is an alias for -Hostname in newer modules {#sec8}
Connect-OVMgmt -Appliance oneview.example.com -Credential $cred

## How the Automation Selects Modules

- Explicit config: oneview_config.json → module_name

- Auto-detect: scans installed modules, picks highest version

- Fallback: defaults to HPEOneView.1000 if none found

## Module Validation

When Set-MaintenanceMode runs (non-dry-run) it validates: - Module exists on the target system - PowerShell version compatibility (warns if PS 7+ required but unavailable) - Logs the selected module name

## Related Documentation

- HPE OneView POSH Library

- HPE OneView PowerShell Samples

- Module Documentation

- HPE OneView PowerShell Module Docs